# PROJECT TITLE:

# HALO (Heatstroke Alert & Life-saving Organizer)

## TEAM MEMBERS:

·   	Sivaarunmani G K

·   	Haritha K

·   	Sanjaay Manikandan M

·   	Sangamithra B

### List of hardware components used:

·       Breadboard

·       DHT 22 -Humidity and Temperature Sensor

·       G-MOD:WS2812B Addressable RGB LED RING [37 LED]

·       Glyph C6

·       LDR (5mm Light Dependent Resistor)

·       SW420 Vibration Sensor

### Pin configuration and circuit connections:

1. DHT22 (Temperature + Humidity Sensor)

VCC → 3.3V

GND GND

DATA → GPIO14 (DHTPIN in code)

(Use a 10k pull-up resistor between DATA and VCC for stable signal.)

2. LDR (Light Sensor)

If you're using just LDR + resistor (voltage divider):

One leg of LDR → 3.3V

Other leg of LDR → GPIO1 (LDR_PIN in code)

That same leg also → Resistor (10k) → GND

GPIO1 reads the analog voltage (0-3.3V) depending on light.

3. Vibration Sensor (SW-420 or similar)

If it's a digital vibration sensor module:

VCC → 3.3V

GND GND

OUT → GPIO20 (VIB_PIN in code)

### Circuit design/schematic diagrams:

### Code :

#include <WiFi.h>

#include <HTTPClient.h>

#include <Wire.h>

#include <DHT.h>

#include <ArduinoJson.h>

#include <Adafruit_NeoPixel.h>

const char* WIFI_SSID = "Haritha";

const char* WIFI_PASS = "haritha.k";

const char* FIREBASE_HOST = "https://halo-16bfd-default-rtdb.asia-southeast1.firebasedatabase.app/";

const char* FIREBASE_AUTH = "etMqyyz3AsfjbXaDPkPAcErHER9PjRgJEJW3nlXI";

const unsigned long UPLOAD_INTERVAL = 3000; // 3 sec

#define DHTPIN 14

#define DHTTYPE DHT22

#define VIB_PIN 20

#define LDR_PIN 1

#define LED_PIN 8

#define NUM_LEDS 37

DHT dht(DHTPIN, DHTTYPE);

Adafruit_NeoPixel ring(NUM_LEDS, LED_PIN, NEO_GRB + NEO_KHZ800);

unsigned long lastUpload = 0;

String firebasePut(const String &path, const String &jsonPayload) {

String url = String(FIREBASE_HOST) + String(path);

if (String(FIREBASE_AUTH).length() > 0) url += "?auth=" + String(FIREBASE_AUTH);

HTTPClient http;

http.begin(url);

http.addHeader("Content-Type", "application/json");

int httpCode = http.PUT(jsonPayload);

String payload = (httpCode > 0) ? http.getString() : "HTTP_ERROR_" + String(httpCode);

http.end();

return payload;

}

void setRingColor(uint8_t r, uint8_t g, uint8_t b) {

for (int i = 0; i < NUM_LEDS; i++) {

ring.setPixelColor(i, ring.Color(r, g, b));

}

ring.show();

}

void setup() {

Serial.begin(115200);

delay(100);

Serial.println("Glyph C6 monitor starting...");

dht.begin();

delay(2000);

pinMode(VIB_PIN, INPUT_PULLUP);

analogReadResolution(12);

ring.begin();

ring.setBrightness(80);

setRingColor(0, 0, 0);

WiFi.begin(WIFI_SSID, WIFI_PASS);

Serial.print("Connecting to WiFi");

unsigned long tstart = millis();

while (WiFi.status() != WL_CONNECTED && millis() - tstart < 20000) {

delay(500); Serial.print(".");

}

Serial.println(WiFi.status() == WL_CONNECTED ? "\nWiFi connected" : "\nWiFi failed!");

lastUpload = millis();

}

void loop() {

unsigned long now = millis();

float tempC = dht.readTemperature();

float hum = dht.readHumidity();

int ldrValue = analogRead(LDR_PIN);

float ldrPercent = (ldrValue / 4095.0) * 100.0;

bool vib = (digitalRead(VIB_PIN) == LOW);

if (isnan(tempC) || isnan(hum)) {

Serial.println("DHT reading failed!");

return;

}

String status = "safe";

if (tempC > 38 || (tempC > 34 && hum > 75)) {

status = "danger";

setRingColor(255, 0, 0);

}

else if ((tempC > 32 && tempC <= 38) || (tempC > 30 && hum > 65)) {

status = "warning";

setRingColor(255, 80, 0);

}

else {

status = "safe";

setRingColor(0, 150, 0);

}

Serial.println("----- SENSOR DATA -----");

Serial.print("Temperature (C): "); Serial.println(tempC);

Serial.print("Humidity (%): "); Serial.println(hum);

Serial.print("LDR Raw: "); Serial.print(ldrValue);

Serial.print(" ("); Serial.print(ldrPercent); Serial.println("%)");

Serial.print("Vibration: "); Serial.println(vib ? "active" : "inactive");

Serial.print("Status: "); Serial.println(status);

Serial.println("-----------------------");

if (now - lastUpload >= UPLOAD_INTERVAL) {

lastUpload = now;

Serial.println("Uploading to Firebase...");

Serial.println(firebasePut("/sensor/temperature.json", String(tempC)));

Serial.println(firebasePut("/sensor/humidity.json", String(hum)));

Serial.println(firebasePut("/sensor/sunlight.json", String(ldrPercent)));

Serial.println(firebasePut("/sensor/vibration.json", vib ? "\"active\"" : "\"inactive\""));

Serial.println(firebasePut("/sensor/status.json", "\"" + status + "\""));

}

delay(1000);

}

### Software/tools used:

·       Arduino IDE

### Clear pictures of the setup/prototype:

### Working explanation and output:

1. Power & Ground

·       All modules (DHT22, LDR, SW-420, WS2812) share common GND.

·       Use 3.3V from the ESP32/Glyph board to power sensors and LEDs (limited number).

2. DHT22 – Temperature & Humidity

·       Reads ambient temperature (`T`) and humidity (`RH`).

·       Computes  heat index (perceived temperature).

·       Minimum read interval ~2 seconds; retry 2–3 times if reading fails (`NaN`).

·       Output → contributes  main risk factor.

3. LDR – Light Sensor

·       Measures ambient light via voltage divider (LDR + 10kΩ resistor).

·       ADC reading normalized `0.0–1.0` → `sunScore`.

·       If `sunScore > 0.6`, add small offset (+2°C) to heat index → accounts for sunlight exposure.

4. SW-420 – Vibration / Movement Sensor

·       Detects movement spikes.

·       Track last movement using `millis()`..

·       If no movement for 5 min → inactivity offset (+1) applied to risk.

5. WS2812 – Visual Indicator

·       LED color shows risk level

·       Green → Safe

·       Yellow → Caution

·       Red → High / Danger

·       Risk level calculated as:

```

·       riskValue = heatIndex + sunOffset + inactivityOffset

```

6. Logic Flow

·       Read DHT22 → heat index (`HIc`).

·       Read LDR → calculate `sunScore` → add sun offset if bright.

·       Read SW-420 → detect movement/inactivity → add inactivity offset.

·       Combine → `riskValue` → map to LED color.

·       Repeat every ~8 seconds for live monitoring.

7. Important Notes

*No RTC needed; `millis()` handles relative time for inactivity.

*Works real-time, suitable for alerts.

* Keep LED count small and brightness low to avoid overloading 3.3V rail

### Applications and other relevant details:

- Heatwave & Climate Safety Monitor

  - Detects extreme heat/humidity using DHT22 → warns users about dehydration risk.

  - Links to global warming awareness (rising heat → rising health risks).

- Personal Health & Safety Band

  - Monitors body/environment conditions.

  - Alerts if fall/shock detected (SW-420 vibration sensor).

- Workplace Safety

  - For outdoor workers, construction staff, farmers: band can signal unsafe temperature + light exposure.

- Disaster Preparedness Gadget

  - Can log data with RTC for post-event analysis.

  - Example: in a heatwave/flood relief camp, volunteers can wear it.

- Smart Energy & Awareness Tool

  - LDR can track sunlight exposure → optimize working hours outside.

  - Encourages people to reduce exposure → connects to climate change mitigation.

- Low-Cost Environmental Logger

  - With WiFi/cloud, can act as a distributed IoT node measuring environment across different areas.

- Smart Home / Daily Use

  - Alerts with LEDs if humidity/temp crosses threshold → useful for elderly/children.

### Other Relevant Details

- Sensors Used & Their Roles

  - DHT22: Accurate temp/humidity (±2% RH, ±0.5°C).

  - LDR: Simple light detection → cheap way to measure sun exposure.

  - SW-420: Motion/fall detection (simple digital trigger).

  - RTC PCF8563: Keeps accurate time for logging events.

  - WS2812 LEDs: Attractive visual alerts (color-coded for severity).

2.        Advantages
 => Low-cost & portable.
 => Works offline (LED alerts) and online (data logging).
 => Combines environmental + safety monitoring.
 => Can be worn → high user interaction & demo appeal.

- Limitations
 => DHT22 has slower response time (sampling every ~2s).
 => SW-420 is noisy → needs software debouncing.
 => Power consumption (WS2812) is high → limit brightness.
 => Breadboard/perfboard version may not be durable → need enclosure.

- Improvements Possible
 => Add GPS → track heat zones.
 => Add WiFi/Firebase logging (Glyph supports it).
 => Replace SW-420 with MPU6050 (more accurate fall detection).
 => Power optimization for longer wearable use.

- Connection to Global Warming / Novelty Angle

  - Rising heatwaves and UV exposure → need personalized alert systems.

  - Your band is like a “climate-aware guardian” on the wrist.

  - Judges will connect emotionally: “Imagine a farmer collapsing under heat stress — this band can warn him early and save lives.”